EXP. NO: 4

MATLAB SIMULATION OF SUPERHETERODYNE RECEIVER FOR AM COMMUNICATION SYSTEM WITH FREQUENCY MIXING AND FILTERING

OBJECTIVE:

i) To model and simulate the operation of a superheterodyne receiver for an AM communication system using MATLAB by implementing frequency mixing and intermediate frequency (IF) signal generation.

PROGRAMMING CODE:

clc;

clear;

close all;

% Parameters

Am = 1;              % Message amplitude

fm = 500;            % Message frequency (Hz)

Ac = 5;              % Carrier amplitude

fRF = 10000;         % RF carrier frequency (Hz)

fLO = 15000;         % Local oscillator frequency (Hz)

fIF = abs(fLO-fRF);  % Intermediate frequency (Hz)

mu = 0.5;            % Modulation index

fs = 200000;         % Sampling frequency

T = 0.02;            % Signal duration

% Time vector

t = 0:1/fs:T-1/fs;

% 1. Generate message signal

m = Am*cos(2*pi*fm*t);

% 2. Generate received AM signal

rf = Ac*(1 + mu*cos(2*pi*fm*t)) ...

.* cos(2*pi*fRF*t);

% 3. Generate local oscillator signal

lo = cos(2*pi*fLO*t);

% 4. Frequency mixing

mixed = rf .* lo;

% 5. Design IF bandpass filter

% Pass frequencies around fIF and reject other components

bp = designfilt('bandpassiir', ...

'FilterOrder', 6, ...

'HalfPowerFrequency1', fIF-1500, ...

'HalfPowerFrequency2', fIF+1500, ...

'SampleRate', fs);

% 6. Generate IF signal

if_signal = filtfilt(bp, mixed);

% Frequency spectrum of IF signal

N = length(t);

f = (-N/2:N/2-1)*(fs/N);

RF_FFT = abs(fftshift(fft(rf)))/N;

MIX_FFT = abs(fftshift(fft(mixed)))/N;

IF_FFT = abs(fftshift(fft(if_signal)))/N;

% 7. Plot results

figure('Name','Superheterodyne AM Receiver', ...

'NumberTitle','off');

subplot(3,2,1);

plot(t,m);

title('Original Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,2,2);

plot(t,rf);

title('Received AM Signal (RF)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,2,3);

plot(t,lo);

title('Local Oscillator Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,2,4);

plot(t,mixed);

title('Mixer Output');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,2,5);

plot(t,if_signal);

title('Filtered Intermediate Frequency Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,2,6);

plot(f,IF_FFT);

xlim([0 12000]);

title('IF Signal Frequency Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

grid on;

% Display calculated frequencies

fprintf('RF Frequency = %d Hz\n',fRF);

fprintf('LO Frequency = %d Hz\n',fLO);

fprintf('Intermediate Frequency = %d Hz\n',fIF);

fprintf('Superheterodyne receiver simulation completed.\n');

OUTPUT:

OBJECTIVE 2 :

ii) To recover the original message signal through intermediate frequency filtering and envelope detection, and analyze the waveforms of the message, carrier, AM, IF, and recovered signals.

Programming Code:

clc;

clear;

close all;

% Parameters

Am = 1;                 % Message amplitude

fm = 500;               % Message frequency (Hz)

Ac = 5;                 % Carrier amplitude

fRF = 10000;             % RF carrier frequency (Hz)

fLO = 15000;             % Local oscillator frequency (Hz)

fIF = abs(fLO-fRF);      % Intermediate frequency (Hz)

mu = 0.5;                % Modulation index

fs = 200000;             % Sampling frequency

T = 0.02;                % Signal duration

% Time vector

t = 0:1/fs:T-1/fs;

N = length(t);

% 1. Generate message signal

m = Am*cos(2*pi*fm*t);

% 2. Generate carrier signal

carrier = Ac*cos(2*pi*fRF*t);

% 3. Generate AM signal

am = Ac*(1 + mu*cos(2*pi*fm*t)) ...

.* cos(2*pi*fRF*t);

% 4. Generate local oscillator signal

lo = cos(2*pi*fLO*t);

% 5. Frequency mixing

mixed = am .* lo;

% 6. Design IF bandpass filter

bp = designfilt('bandpassiir', ...

'FilterOrder', 6, ...

'HalfPowerFrequency1', fIF-1500, ...

'HalfPowerFrequency2', fIF+1500, ...

'SampleRate', fs);

% 7. Extract IF signal

if_signal = filtfilt(bp, mixed);

% 8. Envelope detection

envelope = abs(hilbert(if_signal));

% Remove DC component

recovered = envelope - mean(envelope);

% Normalize recovered message

recovered = recovered / max(abs(recovered));

% 9. Calculate frequency spectrum

f = (-floor(N/2):ceil(N/2)-1)*(fs/N);

IF_FFT = abs(fftshift(fft(if_signal)))/N;

% 10. Plot waveforms

figure('Name','Superheterodyne AM Receiver Analysis', ...

'NumberTitle','off');

subplot(3,2,1);

plot(t,m);

title('Original Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,2,2);

plot(t,carrier);

title('Carrier Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,2,3);

plot(t,am);

title('AM Modulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,2,4);

plot(t,if_signal);

title('Filtered IF Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,2,5);

plot(t,m,'b','LineWidth',1.2);

hold on;

plot(t,recovered,'r--','LineWidth',1.2);

title('Original and Recovered Message');

xlabel('Time (s)');

ylabel('Normalized Amplitude');

legend('Original Message','Recovered Message');

grid on;

hold off;

subplot(3,2,6);

plot(f,IF_FFT);

xlim([fIF-2*fm fIF+2*fm]);

title('IF Frequency Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

grid on;

% 11. Display results

fprintf('RF Frequency = %d Hz\n',fRF);

fprintf('LO Frequency = %d Hz\n',fLO);

fprintf('IF Frequency = %d Hz\n',fIF);

fprintf('Message Frequency = %d Hz\n',fm);

fprintf('Envelope detection completed successfully.\n');

OUTPUT: