EXP. NO: 6

GENERATION AND ANALYSIS OF FREQUENCY MODULATED (FM) AND OF PHASE MODULATED (PM) SIGNALS USING MATLAB

OBJECTIVE:

To generate and analyze Frequency Modulated (FM) and Phase Modulated (PM) signals using

MATLAB by varying the instantaneous frequency and phase of a carrier signal in accordance

with the message signal.

PROGRAMMING CODE:

clc;

clear;

close all;

% Parameters

Am = 1;             % Message amplitude

fm = 100;           % Message frequency (Hz)

Ac = 2;             % Carrier amplitude

fc = 1000;          % Carrier frequency (Hz)

fs = 100000;        % Sampling frequency

T = 0.05;           % Signal duration

kf = 200;           % FM frequency sensitivity (Hz/unit)

kp = pi/2;          % PM phase sensitivity (rad/unit)

% Time vector

t = 0:1/fs:T-1/fs;

% 1. Generate message signal

m = Am*cos(2*pi*fm*t);

% 2. Generate carrier signal

carrier = Ac*cos(2*pi*fc*t);

% 3. Frequency Modulation

% Integral of message signal

int_m = cumtrapz(t,m);

% FM signal

fm_signal = Ac*cos(2*pi*fc*t + 2*pi*kf*int_m);

% Instantaneous frequency of FM

f_inst = fc + kf*m;

% 4. Phase Modulation

pm_signal = Ac*cos(2*pi*fc*t + kp*m);

% Instantaneous phase of PM

phase_pm = 2*pi*fc*t + kp*m;

% Instantaneous frequency of PM

f_pm_inst = fc - kp*Am*fm*sin(2*pi*fm*t);

% 5. Plot results

figure('Name','FM and PM Modulation Analysis', ...

'NumberTitle','off');

subplot(4,2,1);

plot(t,m);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,2,2);

plot(t,carrier);

title('Carrier Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,2,3);

plot(t,fm_signal);

title('Frequency Modulated (FM) Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,2,4);

plot(t,f_inst);

title('FM Instantaneous Frequency');

xlabel('Time (s)');

ylabel('Frequency (Hz)');

grid on;

subplot(4,2,5);

plot(t,pm_signal);

title('Phase Modulated (PM) Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,2,6);

plot(t,phase_pm);

title('PM Instantaneous Phase');

xlabel('Time (s)');

ylabel('Phase (rad)');

grid on;

subplot(4,2,7);

plot(t,f_pm_inst);

title('PM Instantaneous Frequency');

xlabel('Time (s)');

ylabel('Frequency (Hz)');

grid on;

subplot(4,2,8);

plot(t,m,'b','LineWidth',1.2);

hold on;

plot(t,f_inst-fc,'r--','LineWidth',1.2);

title('Message vs FM Frequency Deviation');

xlabel('Time (s)');

ylabel('Amplitude / Frequency Deviation');

legend('Message','FM Deviation');

grid on;

hold off;

% 6. Display results

fprintf('FM and PM Modulation Analysis\n');

fprintf('-----------------------------\n');

fprintf('Message Frequency = %d Hz\n',fm);

fprintf('Carrier Frequency = %d Hz\n',fc);

fprintf('FM Frequency Deviation = %.2f Hz\n',kf*Am);

fprintf('FM Modulation Index = %.2f\n',(kf*Am)/fm);

fprintf('PM Phase Deviation = %.2f radians\n',kp*Am);

fprintf('FM Frequency Range = %.2f to %.2f Hz\n', ...

fc-kf*Am,fc+kf*Am);

fprintf('FM and PM signals generated successfully.\n');

OUTPUT: