EXP. NO: 10

PERFORMANCE ANALYSIS OF FM SYSTEM UNDER GAUSSIAN WHITE NOISE USING MATLAB

Objectives:

1. To generate a Frequency Modulated (FM) signal in MATLAB using the specified carrier and

message signal parameters.

PROGRAMMING CODE:

clc;

clear;

close all;

% Given Parameters

Am = 1;          % Message signal amplitude

Ac = 2;          % Carrier signal amplitude

fm = 100;        % Message frequency in Hz

fc = 1000;       % Carrier frequency in Hz

kf = 500;        % Frequency sensitivity in Hz/V

% Sampling Parameters

fs = 10000;      % Sampling frequency in Hz

T = 0.05;        % Duration in seconds

t = 0:1/fs:T-1/fs;

% Generate Message Signal

m = Am*cos(2*pi*fm*t);

% Generate Carrier Signal

c = Ac*cos(2*pi*fc*t);

% Calculate Frequency Deviation

delta_f = kf*Am;

% Calculate Modulation Index

beta = delta_f/fm;

% Generate FM Signal

s = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

% Display Results

fprintf('Carrier Frequency = %.2f Hz\n',fc);

fprintf('Message Frequency = %.2f Hz\n',fm);

fprintf('Frequency Deviation = %.2f Hz\n',delta_f);

fprintf('FM Modulation Index = %.2f\n',beta);

% Plot Signals

figure;

subplot(3,1,1);

plot(t,m);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,1,2);

plot(t,c);

title('Carrier Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,1,3);

plot(t,s);

title('Frequency Modulated (FM) Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

OUTPUTS:

OBJECTIVE:

2. To introduce Additive White Gaussian Noise (AWGN) into the generated FM signal and simulate the effect of a noisy communication channel using MATLAB.

PROGRAMMING CODE:

clc;

clear;

close all;

% Given Parameters

Am = 1;          % Message signal amplitude

Ac = 2;          % Carrier amplitude

fm = 100;        % Message frequency in Hz

fc = 1000;       % Carrier frequency in Hz

kf = 500;        % Frequency sensitivity in Hz/V

SNR = 10;        % Signal-to-Noise Ratio in dB

% Sampling Parameters

fs = 10000;      % Sampling frequency in Hz

T = 0.05;        % Duration in seconds

t = 0:1/fs:T-1/fs;

% Generate Message Signal

m = Am*cos(2*pi*fm*t);

% Calculate FM Parameters

delta_f = kf*Am;

beta = delta_f/fm;

% Generate FM Signal

s = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

% Calculate Signal Power

signal_power = mean(s.^2);

% Generate AWGN

noise_power = signal_power/(10^(SNR/10));

noise = sqrt(noise_power)*randn(size(s));

% Received FM Signal

r = s + noise;

% Calculate Actual SNR

measured_SNR = 10*log10(mean(s.^2)/mean(noise.^2));

% Display Results

fprintf('Carrier Frequency = %.2f Hz\n',fc);

fprintf('Message Frequency = %.2f Hz\n',fm);

fprintf('Frequency Deviation = %.2f Hz\n',delta_f);

fprintf('Modulation Index = %.2f\n',beta);

fprintf('Specified SNR = %.2f dB\n',SNR);

fprintf('Measured SNR = %.2f dB\n',measured_SNR);

% Plot Signals

figure;

subplot(4,1,1);

plot(t,m);

title('Original Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,1,2);

plot(t,s);

title('Generated FM Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,1,3);

plot(t,noise);

title('Additive White Gaussian Noise');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,1,4);

plot(t,r);

title(['Received FM Signal with AWGN (SNR = ',num2str(SNR),' dB)']);

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

OUTPUTS:

OBJECTIVE:

3. To analyze the effect of AWGN on the transmitted FM signal by comparing the original and noisy FM signals using time-domain and frequency-domain analysis.

PROGRAMMING CODE:

clc;

clear;

close all;

% Parameters

Am = 1;           % Message signal amplitude

Ac = 2;           % Carrier amplitude

fm = 100;         % Message frequency in Hz

fc = 1000;        % Carrier frequency in Hz

kf = 500;         % Frequency sensitivity in Hz/V

SNR = 10;         % Signal-to-noise ratio in dB

% Sampling parameters

fs = 10000;

T = 0.1;

t = 0:1/fs:T-1/fs;

% Generate message signal

m = Am*cos(2*pi*fm*t);

% Calculate FM parameters

delta_f = kf*Am;

beta = delta_f/fm;

% Generate original FM signal

s = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

% Add AWGN

Ps = mean(s.^2);

Pn = Ps/10^(SNR/10);

noise = sqrt(Pn)*randn(size(s));

% Received noisy FM signal

r = s + noise;

% Calculate measured SNR

measured_SNR = 10*log10(mean(s.^2)/mean(noise.^2));

% FFT analysis

N = length(s);

f = (0:N/2)*(fs/N);

S = fft(s);

R = fft(r);

% Single-sided amplitude spectra

S_mag = abs(S(1:N/2+1))/N;

R_mag = abs(R(1:N/2+1))/N;

S_mag(2:end-1) = 2*S_mag(2:end-1);

R_mag(2:end-1) = 2*R_mag(2:end-1);

% Display results

fprintf('Frequency Deviation = %.2f Hz\n',delta_f);

fprintf('Modulation Index = %.2f\n',beta);

fprintf('Specified SNR = %.2f dB\n',SNR);

fprintf('Measured SNR = %.2f dB\n',measured_SNR);

% Time-domain analysis

figure;

subplot(3,1,1);

plot(t,s);

title('Original FM Signal');

xlabel('Time (s)');

ylabel('Amplitude');

xlim([0 0.02]);

grid on;

subplot(3,1,2);

plot(t,r);

title('Noisy FM Signal');

xlabel('Time (s)');

ylabel('Amplitude');

xlim([0 0.02]);

grid on;

subplot(3,1,3);

plot(t,s,'b');

hold on;

plot(t,r,'r');

title('Original and Noisy FM Signal Comparison');

xlabel('Time (s)');

ylabel('Amplitude');

legend('Original FM','Noisy FM');

xlim([0 0.02]);

grid on;

% Frequency-domain analysis

figure;

subplot(2,1,1);

plot(f,S_mag);

title('Spectrum of Original FM Signal');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

xlim([0 2000]);

grid on;

subplot(2,1,2);

plot(f,R_mag);

title('Spectrum of Noisy FM Signal');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

xlim([0 2000]);

grid on;

OUTPUT: