EXP. NO: 16

SIMULATION AND ANALYSIS OF UNIFORM AND NON-UNIFORM QUANTIZATION USING MATLAB

Objectives

i) To implement and analyze uniform and non-uniform quantization techniques for an analog

signal using MATLAB by converting the continuous-amplitude signal into discrete

quantization levels.

PROGRAMMING CODE:

clc;

clear;

close all;

% Parameters

Am = 1;             % Signal amplitude

fm = 5;             % Signal frequency in Hz

fs = 1000;           % Sampling frequency in Hz

T = 1;               % Signal duration in seconds

n = 3;               % Number of quantization bits

L = 2^n;             % Number of quantization levels

% Generate Analog Signal

t = 0:1/fs:T;

x = Am*sin(2*pi*fm*t);

% ---------------- UNIFORM QUANTIZATION ----------------

xmin = -Am;

xmax = Am;

Delta = (xmax-xmin)/L;     % Quantization step size

% Uniform quantization

x_uniform = xmin + Delta/2 + ...

Delta*floor((x-xmin)/Delta);

% Limit output to maximum quantization level

x_uniform(x_uniform > xmax-Delta/2) = xmax-Delta/2;

% ---------------- NON-UNIFORM QUANTIZATION ----------------

% Mu-law compression parameter

mu = 255;

% Normalize input signal

x_norm = x/Am;

% Mu-law compression

x_compressed = sign(x_norm).* ...

log(1+mu*abs(x_norm))/log(1+mu);

% Quantize compressed signal uniformly

Delta_mu = 2/L;

xq_compressed = -1 + Delta_mu/2 + ...

Delta_mu*floor((x_compressed+1)/Delta_mu);

% Limit quantized values

xq_compressed(xq_compressed > 1-Delta_mu/2) = ...

1-Delta_mu/2;

% Expand the quantized signal

x_nonuniform = sign(xq_compressed).* ...

((1+mu).^abs(xq_compressed)-1)/mu;

% Restore original amplitude

x_nonuniform = Am*x_nonuniform;

% ---------------- PLOTTING ----------------

figure;

subplot(3,1,1);

plot(t,x,'b','LineWidth',1.5);

title('Original Analog Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,1,2);

stairs(t,x_uniform,'r','LineWidth',1.2);

hold on;

plot(t,x,'b--');

title('Uniform Quantization');

xlabel('Time (s)');

ylabel('Amplitude');

legend('Quantized Signal','Original Signal');

grid on;

subplot(3,1,3);

stairs(t,x_nonuniform,'m','LineWidth',1.2);

hold on;

plot(t,x,'b--');

title('Non-Uniform Quantization using Mu-Law');

xlabel('Time (s)');

ylabel('Amplitude');

legend('Quantized Signal','Original Signal');

grid on;

% ---------------- ERROR CALCULATION ----------------

error_uniform = x-x_uniform;

error_nonuniform = x-x_nonuniform;

MSE_uniform = mean(error_uniform.^2);

MSE_nonuniform = mean(error_nonuniform.^2);

fprintf('Number of Bits = %d\n',n);

fprintf('Number of Quantization Levels = %d\n',L);

fprintf('Uniform Quantization Step Size = %.4f\n',Delta);

fprintf('Uniform Quantization MSE = %.6f\n',MSE_uniform);

fprintf('Non-Uniform Quantization MSE = %.6f\n',MSE_nonuniform);

OUTPUT:

OBJECTIVE:

ii) To compare the performance of uniform and non-uniform quantization by analyzing their

quantized waveforms and quantization errors, and evaluate their effectiveness in representing

signals with different amplitude variations.

PROGRAMMING CODE:

clc;

clear;

close all;

% EXPERIMENT NO: 16

% Comparison of Uniform and Non-Uniform Quantization

% Signal Parameters

Am = 1;              % Amplitude of the signal

fm = 5;              % Message frequency in Hz

fs = 1000;           % Sampling frequency

T = 1;               % Duration in seconds

n = 3;               % Number of bits

L = 2^n;             % Number of quantization levels

% Generate Input Signal

t = 0:1/fs:T;

x = Am*sin(2*pi*fm*t);

% -------------------------------

% 1. UNIFORM QUANTIZATION

% -------------------------------

xmin = -Am;

xmax = Am;

Delta = (xmax-xmin)/L;

x_uniform = xmin + Delta/2 + ...

Delta*floor((x-xmin)/Delta);

x_uniform(x_uniform > xmax-Delta/2) = xmax-Delta/2;

% Uniform Quantization Error

error_uniform = x-x_uniform;

% -------------------------------

% 2. NON-UNIFORM QUANTIZATION

% Using Mu-Law Companding

% -------------------------------

mu = 255;

% Normalize the input signal

x_norm = x/Am;

% Mu-law compression

x_compressed = sign(x_norm).* ...

log(1+mu*abs(x_norm))/log(1+mu);

% Quantize the compressed signal

Delta_mu = 2/L;

xq_compressed = -1 + Delta_mu/2 + ...

Delta_mu*floor((x_compressed+1)/Delta_mu);

xq_compressed(xq_compressed > 1-Delta_mu/2) = ...

1-Delta_mu/2;

% Expand the quantized signal

x_nonuniform = sign(xq_compressed).* ...

((1+mu).^abs(xq_compressed)-1)/mu;

x_nonuniform = Am*x_nonuniform;

% Non-Uniform Quantization Error

error_nonuniform = x-x_nonuniform;

% -------------------------------

% 3. CALCULATE PERFORMANCE

% -------------------------------

MSE_uniform = mean(error_uniform.^2);

MSE_nonuniform = mean(error_nonuniform.^2);

fprintf('Number of Quantization Levels = %d\n', L);

fprintf('Uniform Quantization Step Size = %.4f\n', Delta);

fprintf('Uniform Quantization MSE = %.6f\n', MSE_uniform);

fprintf('Non-Uniform Quantization MSE = %.6f\n', MSE_nonuniform);

% -------------------------------

% 4. PLOT QUANTIZED WAVEFORMS

% -------------------------------

figure;

subplot(3,1,1);

plot(t,x,'b','LineWidth',1.5);

title('Original Analog Signal');

xlabel('Time (seconds)');

ylabel('Amplitude');

grid on;

subplot(3,1,2);

stairs(t,x_uniform,'r','LineWidth',1.2);

hold on;

plot(t,x,'b--');

title('Uniform Quantization');

xlabel('Time (seconds)');

ylabel('Amplitude');

legend('Quantized Signal','Original Signal');

grid on;

subplot(3,1,3);

stairs(t,x_nonuniform,'m','LineWidth',1.2);

hold on;

plot(t,x,'b--');

title('Non-Uniform Quantization Using Mu-Law');

xlabel('Time (seconds)');

ylabel('Amplitude');

legend('Quantized Signal','Original Signal');

grid on;

% -------------------------------

% 5. PLOT QUANTIZATION ERRORS

% -------------------------------

figure;

subplot(2,1,1);

plot(t,error_uniform,'r','LineWidth',1.2);

title('Uniform Quantization Error');

xlabel('Time (seconds)');

ylabel('Error');

grid on;

subplot(2,1,2);

plot(t,error_nonuniform,'m','LineWidth',1.2);

title('Non-Uniform Quantization Error');

xlabel('Time (seconds)');

ylabel('Error');

grid on;

% -------------------------------

% 6. COMPARE MSE

% -------------------------------

figure;

bar([MSE_uniform MSE_nonuniform]);

set(gca,'XTickLabel',{'Uniform','Non-Uniform'});

title('Mean Square Error Comparison');

ylabel('Mean Square Error (MSE)');

grid on;

OUTPUT: