EXP. NO: 18

MATLAB SIMULATION AND EVALUATION OF EYE DIAGRAM PARAMETERS UNDER INTER-SYMBOL INTERFERENCE IN  DIGITAL COMMUNICATION SYSTEMS

Objectives

1. To simulate a digital communication system in MATLAB and generate the eye diagram

of the received signal for evaluating eye height, eye width, eye-crossing level and timing

margin.

PROGRAMMING CODE:

clc;

clear;

close all;

% EXP. NO: 18

% MATLAB Simulation and Evaluation of Eye Diagram Parameters

% Under Inter-Symbol Interference (ISI)

% 1. SYSTEM PARAMETERS

N = 1000;             % Number of transmitted bits

Rb = 1000;            % Bit rate in bits/second

sps = 100;            % Samples per symbol

fs = Rb*sps;          % Sampling frequency

Ts = 1/Rb;             % Bit duration

SNR = 25;              % Signal-to-noise ratio in dB

% 2. GENERATE RANDOM BINARY DATA

bits = randi([0 1], 1, N);

% Convert bits to bipolar NRZ signal

symbols = 2*bits - 1;

% 3. UPSAMPLING

tx = zeros(1, N*sps);

tx(1:sps:end) = symbols;

% 4. GENERATE CHANNEL WITH INTER-SYMBOL INTERFERENCE

% Simple channel impulse response

h = [0.2 0.6 1 0.6 0.2];

% Normalize channel

h = h/sum(h);

% Introduce ISI through channel convolution

rx_isi = conv(tx, h, 'same');

% 5. ADD AWGN NOISE

signal_power = mean(rx_isi.^2);

noise_power = signal_power/10^(SNR/10);

noise = sqrt(noise_power)*randn(size(rx_isi));

rx = rx_isi + noise;

% 6. PLOT TRANSMITTED AND RECEIVED SIGNALS

t = (0:length(tx)-1)/fs;

figure;

subplot(2,1,1);

plot(t(1:1000), tx(1:1000), 'LineWidth', 1.2);

title('Transmitted NRZ Signal');

xlabel('Time (seconds)');

ylabel('Amplitude');

grid on;

subplot(2,1,2);

plot(t(1:1000), rx(1:1000), 'LineWidth', 1.2);

title('Received Signal with ISI and AWGN');

xlabel('Time (seconds)');

ylabel('Amplitude');

grid on;

% 7. GENERATE EYE DIAGRAM

% Plot 2-bit intervals with 1-bit overlap

eye_length = 2*sps;

num_traces = min(100, floor(length(rx)/sps)-1);

figure;

hold on;

for k = 1:num_traces

start_index = (k-1)*sps + 1;

end_index = start_index + eye_length - 1;

if end_index <= length(rx)

segment = rx(start_index:end_index);

time_eye = (0:eye_length-1)/sps;

plot(time_eye, segment, 'b');

end

end

title('Eye Diagram of Received Signal Under ISI');

xlabel('Time (Bit Periods)');

ylabel('Amplitude');

xlim([0 2]);

grid on;

hold off;

% 8. ESTIMATE EYE PARAMETERS

% Analyze the central sampling instant

center_index = round(sps/2);

% Samples near the center of each bit

sample_indices = center_index:sps:length(rx);

samples = rx(sample_indices);

% Separate samples according to transmitted bits

valid_indices = min(length(bits), length(samples));

samples = samples(1:valid_indices);

valid_bits = bits(1:valid_indices);

zero_samples = samples(valid_bits == 0);

one_samples = samples(valid_bits == 1);

% Estimate decision levels

if ~isempty(zero_samples) && ~isempty(one_samples)

level_zero = mean(zero_samples);

level_one = mean(one_samples);

% Eye crossing / decision threshold

eye_crossing_level = (level_zero + level_one)/2;

% Estimate eye height from the central sampling distributions

lower_one = prctile(one_samples, 5);

upper_zero = prctile(zero_samples, 95);

eye_height = max(0, lower_one - upper_zero);

fprintf('\n------ EYE DIAGRAM PARAMETERS ------\n');

fprintf('Mean level for bit 0 = %.4f\n', level_zero);

fprintf('Mean level for bit 1 = %.4f\n', level_one);

fprintf('Eye crossing / decision threshold = %.4f\n', ...

eye_crossing_level);

fprintf('Estimated eye height = %.4f\n', eye_height);

else

fprintf('Insufficient samples to estimate eye parameters.\n');

end

% 9. ESTIMATE EYE WIDTH AND TIMING MARGIN

% Find the opening around the nominal sampling instant

% using the received waveform and a decision threshold.

if ~isempty(zero_samples) && ~isempty(one_samples)

threshold = eye_crossing_level;

% Analyze the first 100 received bit intervals

num_bits_analyze = min(100, N-2);

phase_values = 1:sps;

valid_phase = false(size(phase_values));

for p = phase_values

idx = p:sps:(num_bits_analyze*sps);

idx = idx(idx <= length(rx));

if isempty(idx)

OUTPUT:

OBJECTIVE:

2. To introduce and study Inter-Symbol Interference (ISI) under different channel

conditions, and evaluate its effect on eye-diagram opening and overall digital signal

quality.

PROGRAMMING CODE:

clc;

clear;

close all;

% EXP. NO: 18

% Study of ISI Under Different Channel Conditions

% 1. SYSTEM PARAMETERS

N = 1000;                 % Number of bits

Rb = 1000;                % Bit rate

sps = 100;                % Samples per bit

fs = Rb*sps;              % Sampling frequency

SNR = 25;                 % Signal-to-noise ratio in dB

% 2. GENERATE RANDOM BINARY DATA

bits = randi([0 1],1,N);

% Convert binary data into bipolar NRZ levels

symbols = 2*bits-1;

% Upsample the transmitted symbols

tx = zeros(1,N*sps);

tx(1:sps:end) = symbols;

% 3. DEFINE DIFFERENT CHANNEL CONDITIONS

% Ideal channel

h1 = 1;

% Channel with moderate ISI

h2 = [0.2 0.6 1 0.6 0.2];

% Channel with severe ISI

h3 = [0.4 0.8 1 0.8 0.4];

% Normalize channel responses

h2 = h2/sum(h2);

h3 = h3/sum(h3);

% 4. PASS SIGNAL THROUGH DIFFERENT CHANNELS

rx1 = conv(tx,h1,'same');

rx2 = conv(tx,h2,'same');

rx3 = conv(tx,h3,'same');

% 5. ADD AWGN NOISE TO EACH RECEIVED SIGNAL

rx1 = addNoise(rx1,SNR);

rx2 = addNoise(rx2,SNR);

rx3 = addNoise(rx3,SNR);

% 6. PLOT RECEIVED SIGNALS

t = (0:length(tx)-1)/fs;

display_samples = min(1000,length(tx));

figure;

subplot(3,1,1);

plot(t(1:display_samples),rx1(1:display_samples));

title('Received Signal - Ideal Channel');

xlabel('Time (seconds)');

ylabel('Amplitude');

grid on;

subplot(3,1,2);

plot(t(1:display_samples),rx2(1:display_samples));

title('Received Signal - Moderate ISI');

xlabel('Time (seconds)');

ylabel('Amplitude');

grid on;

subplot(3,1,3);

plot(t(1:display_samples),rx3(1:display_samples));

title('Received Signal - Severe ISI');

xlabel('Time (seconds)');

ylabel('Amplitude');

grid on;

% 7. GENERATE EYE DIAGRAMS

signals = {rx1,rx2,rx3};

titles = {'Ideal Channel','Moderate ISI','Severe ISI'};

figure;

for c = 1:3

subplot(3,1,c);

hold on;

rx = signals{c};

eye_length = 2*sps;

num_traces = min(100,floor((length(rx)-eye_length)/sps));

for k = 1:num_traces

start_index = (k-1)*sps+1;

end_index = start_index+eye_length-1;

if end_index <= length(rx)

segment = rx(start_index:end_index);

time_eye = (0:eye_length-1)/sps;

plot(time_eye,segment);

end

end

title(['Eye Diagram - ',titles{c}]);

xlabel('Time (Bit Periods)');

ylabel('Amplitude');

xlim([0 2]);

grid on;

hold off;

end

% 8. CALCULATE BIT ERROR RATE

BER = zeros(1,3);

signals = {rx1,rx2,rx3};

for c = 1:3

rx = signals{c};

% Sample near the center of each bit

sample_indices = round(sps/2):sps:length(rx);

sample_indices = sample_indices(sample_indices <= length(rx));

% Align sample positions with transmitted bits

received_samples = rx(sample_indices);

detected_bits = received_samples > 0;

valid_bits = min(length(bits),length(detected_bits));

BER(c) = sum(bits(1:valid_bits) ~= ...

detected_bits(1:valid_bits))/valid_bits;

end

% 9. DISPLAY RESULTS

fprintf('\n----- ISI PERFORMANCE COMPARISON -----\n');

fprintf('Ideal Channel BER     = %.4f\n',BER(1));

fprintf('Moderate ISI BER      = %.4f\n',BER(2));

fprintf('Severe ISI BER        = %.4f\n',BER(3));

figure;

bar(BER);

set(gca,'XTickLabel',{'Ideal Channel','Moderate ISI','Severe ISI'});

ylabel('Bit Error Rate (BER)');

title('Effect of Channel ISI on Bit Error Rate');

grid on;

% 10. LOCAL FUNCTION TO ADD AWGN

function y = addNoise(x,SNR)

signal_power = mean(x.^2);

noise_power = signal_power/10^(SNR/10);

noise = sqrt(noise_power)*randn(size(x));

y = x+noise;

end

OUTPUT: