EX NO: 7

CALCULATION AND VERIFICATION OF FREQUENCY DEVIATION AND MODULATION INDEX OF FM SIGNALS USING MATLAB SIMULATION

Objectives:

1. To generate a Frequency Modulated (FM) signal in MATLAB using specified carrier and

message signal parameters.

Programming Code:

clc;

clear;

close all;

% Parameters

Am = 1;          % Message signal amplitude

Ac = 2;          % Carrier signal amplitude

fm = 100;        % Message frequency in Hz

fc = 1000;       % Carrier frequency in Hz

beta = 5;        % Modulation index

% Time vector

fs = 100000;     % Sampling frequency

t = 0:1/fs:0.02; % Time duration

% Message signal

m = Am * cos(2*pi*fm*t);

% Carrier signal

c = Ac * cos(2*pi*fc*t);

% Frequency Modulated signal

s = Ac * cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

% Plot signals

figure;

subplot(3,1,1);

plot(t,m);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,1,2);

plot(t,c);

title('Carrier Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,1,3);

plot(t,s);

title('Frequency Modulated (FM) Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

OUTPUT:

OBJECTIVE:

2. To calculate and verify the frequency deviation of the FM signal from the given carrier

frequency and modulating signal parameters.

PROGRAMMING CODE:

clc;

clear;

close all;

% Given Parameters

Am = 1;          % Message signal amplitude

Ac = 2;          % Carrier signal amplitude

fm = 100;        % Message frequency in Hz

fc = 1000;       % Carrier frequency in Hz

kf = 500;        % Frequency sensitivity in Hz/V

% Calculate Frequency Deviation

delta_f = kf * Am;

% Display Results

fprintf('Carrier Frequency = %d Hz\n', fc);

fprintf('Message Frequency = %d Hz\n', fm);

fprintf('Frequency Sensitivity = %d Hz/V\n', kf);

fprintf('Maximum Frequency Deviation = %d Hz\n', delta_f);

% Time Vector

fs = 100000;

t = 0:1/fs:0.05;

% Generate Message Signal

m = Am * cos(2*pi*fm*t);

% Generate FM Signal

s = Ac * cos(2*pi*fc*t + 2*pi*kf*Am/(2*pi*fm)*sin(2*pi*fm*t));

% Calculate Instantaneous Frequency

f_inst = fc + kf * m;

% Plot Message Signal

figure;

subplot(3,1,1);

plot(t,m);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

% Plot FM Signal

subplot(3,1,2);

plot(t,s);

title('Frequency Modulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

% Plot Instantaneous Frequency

subplot(3,1,3);

plot(t,f_inst);

title('Instantaneous Frequency of FM Signal');

xlabel('Time (s)');

ylabel('Frequency (Hz)');

grid on;

OBJECTIVE:

3. To determine and verify the modulation index of the FM signal using the relationship

between frequency deviation and message frequency.

PROGRAMMING CODE:

clc;

clear;

close all;

% Given Parameters

Am = 1;          % Message signal amplitude

Ac = 2;          % Carrier signal amplitude

fm = 100;        % Message frequency in Hz

fc = 1000;       % Carrier frequency in Hz

kf = 500;        % Frequency sensitivity in Hz/V

% Calculate Frequency Deviation

delta_f = kf * Am;

% Calculate Modulation Index

beta = delta_f / fm;

% Display Results

fprintf('Carrier Frequency = %d Hz\n', fc);

fprintf('Message Frequency = %d Hz\n', fm);

fprintf('Maximum Frequency Deviation = %d Hz\n', delta_f);

fprintf('FM Modulation Index = %.2f\n', beta);

% Time Vector

fs = 100000;

t = 0:1/fs:0.05;

% Generate Message Signal

m = Am * cos(2*pi*fm*t);

% Generate Carrier Signal

c = Ac * cos(2*pi*fc*t);

% Generate FM Signal

s = Ac * cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

% Plot Signals

figure;

subplot(3,1,1);

plot(t,m);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,1,2);

plot(t,c);

title('Carrier Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,1,3);

plot(t,s);

title('Frequency Modulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

OUTPUT:

OBJECTIVE:

4. To analyze the effect of message frequency on frequency deviation and modulation

index, and verify their relationship through MATLAB simulation

PROGRAMMING CODE:

clc;

clear;

close all;

% Parameters

Am = 1;                  % Message signal amplitude

Ac = 2;                  % Carrier signal amplitude

fc = 1000;               % Carrier frequency in Hz

kf = 500;                % Frequency sensitivity in Hz/V

% Different message frequencies

fm = [50 100 200 300 400];

% Calculate frequency deviation and modulation index

delta_f = kf * Am * ones(size(fm));

beta = delta_f ./ fm;

% Display results

fprintf('Message Frequency (Hz)\tFrequency Deviation (Hz)\tModulation Index\n');

fprintf('---------------------------------------------------------------\n');

for i = 1:length(fm)

fprintf('%10d\t\t%15.2f\t\t%10.2f\n', ...

fm(i), delta_f(i), beta(i));

end

% Plot frequency deviation

figure;

subplot(2,1,1);

plot(fm, delta_f, 'bo-', 'LineWidth', 2);

title('Message Frequency vs Frequency Deviation');

xlabel('Message Frequency (Hz)');

ylabel('Frequency Deviation (Hz)');

grid on;

% Plot modulation index

subplot(2,1,2);

plot(fm, beta, 'ro-', 'LineWidth', 2);

title('Message Frequency vs Modulation Index');

xlabel('Message Frequency (Hz)');

ylabel('Modulation Index');

grid on;

% Generate FM signals for different message frequencies

fs = 100000;

t = 0:1/fs:0.02;

figure;

for i = 1:length(fm)

beta_i = delta_f(i) / fm(i);

s = Ac * cos(2*pi*fc*t + beta_i*sin(2*pi*fm(i)*t));

subplot(length(fm),1,i);

plot(t,s);

title(['FM Signal for Message Frequency = ', num2str(fm(i)), ' Hz']);

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

end

OUTPUT: