EXP. NO: 9

PERFORMANCE EVALUATION OF AM GENERATION AND DETECTION UNDER NOISY CHANNEL CONDITIONS USING MATLAB

Objectives:

i) To generate an Amplitude Modulated (AM) signal, introduce Additive White Gaussian Noise

(AWGN), and analyze the effect of noise on the transmitted signal using MATLAB.

PROGRAMMING CODE:

clc;

clear;

close all;

% Given Parameters

Am = 1;          % Message signal amplitude

Ac = 2;          % Carrier signal amplitude

fm = 100;        % Message frequency in Hz

fc = 1000;       % Carrier frequency in Hz

mu = 0.8;        % Modulation index

SNR = 10;        % Signal-to-Noise Ratio in dB

% Sampling Parameters

fs = 10000;      % Sampling frequency in Hz

T = 0.05;        % Duration in seconds

t = 0:1/fs:T-1/fs;

% Generate Message Signal

m = Am*cos(2*pi*fm*t);

% Generate Carrier Signal

c = Ac*cos(2*pi*fc*t);

% Generate AM Signal

s = Ac*(1 + mu*cos(2*pi*fm*t)).*cos(2*pi*fc*t);

% Add AWGN Noise

signal_power = mean(s.^2);

noise_power = signal_power/(10^(SNR/10));

noise = sqrt(noise_power)*randn(size(s));

% Received Signal

r = s + noise;

% Display SNR

measured_SNR = 10*log10(mean(s.^2)/mean(noise.^2));

fprintf('Modulation Index = %.2f\n',mu);

fprintf('Specified SNR = %.2f dB\n',SNR);

fprintf('Measured SNR = %.2f dB\n',measured_SNR);

% Plot Signals

figure;

subplot(4,1,1);

plot(t,m);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,1,2);

plot(t,s);

title('Generated AM Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,1,3);

plot(t,noise);

title('Additive White Gaussian Noise (AWGN)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,1,4);

plot(t,r);

title(['Received AM Signal with AWGN, SNR = ',num2str(SNR),' dB']);

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

OUTPUTS:

OBJECTIVE:

ii) To recover the original message signal from the noisy AM signal using envelope detection

and compare the transmitted and recovered waveforms to evaluate the performance of the AM communication system.

PROGRAMMING CODE:

clc;

clear;

close all;

% Given Parameters

Am = 1;          % Message signal amplitude

Ac = 2;          % Carrier amplitude

fm = 100;        % Message frequency in Hz

fc = 1000;       % Carrier frequency in Hz

mu = 0.8;        % Modulation index

SNR = 10;        % Signal-to-Noise Ratio in dB

% Sampling Parameters

fs = 10000;      % Sampling frequency

T = 0.1;         % Duration in seconds

t = 0:1/fs:T-1/fs;

% Generate Message Signal

m = Am*cos(2*pi*fm*t);

% Generate AM Signal

s = Ac*(1 + mu*cos(2*pi*fm*t)).*cos(2*pi*fc*t);

% Add AWGN

signal_power = mean(s.^2);

noise_power = signal_power/(10^(SNR/10));

noise = sqrt(noise_power)*randn(size(s));

r = s + noise;

% Envelope Detection

envelope = abs(hilbert(r));

% Remove DC Component and Recover Message

recovered = (envelope - mean(envelope))/(Ac*mu);

% Calculate Correlation

correlation = corrcoef(m, recovered);

corr_value = correlation(1,2);

% Calculate Mean Squared Error

mse = mean((m - recovered).^2);

% Display Results

fprintf('SNR = %.2f dB\n', SNR);

fprintf('Correlation Coefficient = %.4f\n', corr_value);

fprintf('Mean Squared Error = %.4f\n', mse);

% Plot Results

figure;

subplot(4,1,1);

plot(t,m,'b','LineWidth',1.2);

title('Original Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,1,2);

plot(t,s,'b');

title('Transmitted AM Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,1,3);

plot(t,r,'r');

title('Received AM Signal with AWGN');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,1,4);

plot(t,m,'b','LineWidth',1.2);

hold on;

plot(t,recovered,'r--','LineWidth',1.2);

title('Comparison of Original and Recovered Message');

xlabel('Time (s)');

ylabel('Amplitude');

legend('Original Message','Recovered Message');

grid on;

OUTPUT: